National School of Business Management

LAB Sheet 08

Lab 08 3 layer classic CISCO design with RIP and redundant links

Set up the following scenario on Packet tracer/physical

Note: - please use the LAB 07 setup as the base and implement the above for the case study given using the classical 3 tier network model

Step1. Complete the full implementation of lab 7

Implement the network model given on lab 07

Step 2. Identify the uplink ports connecting the distribution switchers

- Identify the ethernet ports you are going to use for connecting the PC and the Switch /s.

Step3 Assign IP

- Assign an Interface IP to the relevant ports/logical interfaces  and Ethernet card of the computers

Step4 Testing basic connectivity and RIP behavior

- Use the basic ICMP command and rout prints to identify the routing with redundant links .

- Disable the uplink from the distribution to core

- Use the basic ICMP command and rout prints to identify the routing with redundant links

- Check both routs and routing table and verify the rip is working properly.

You may use the following commands to carry out the task

| No | Disruption | Command |
| --- | --- | --- |
| 01 | IP address for the interface | Ip Address x.x.x.x y.y.y.y |
| 02 | Enable RIP | Router(config)# router rip |
| 03 | Specify Version | Router(config-router)# version 2 |
| 04 | Advertise Networks | Router(config-router)# network [network] |
| 05 | Disable Auto-Summary | Router(config-router)# no auto-summary |
| 06 | Verify | show ip route / show ip protocols |