National School of Business Management

LAB Sheet 06

Lab 06 routing with multilayer switchers

Set up the following scenario on Packet tracer/physical

Note:- please reset  the routers before you start

Step1. Identify the two different ports in the switchers

- Identify the ethernet ports you are going to use for connecting the PC and the Switch /s.

Step2 Assign IP

- Assign an Interface IP to the relevant ports/logical interfaces  and Ethernet card of the computers

Ip interface must be in the following format on IPv4

10.1.X.250 / where the group number in single digit eg group 3 will configure it as 10.1.3.250

Step3 Testing basic connectivity

- Use the basic ICMP command

Step 4 Adding static routes

- The routers need to be configured to handle manual statis and must be tested if it

You may use the following commands to carry out the task

| No | Disruption | command |
| --- | --- | --- |
| 01 | Switch to configuration mode – configuration terminal | Config t |
| 02 | Switch to enable mode of a switch /router | Enable |
| 03 | To go to VLAN configurations | vlan |
| 04 | Change name Name | name |
| 05 | To Access Interface commands | interface |
| 06 | Assign the port to a VLAN. Valid VLAN IDs are 1 to 4094. | switchport access vlan |
| 07 | Define the VLAN membership mode for the port (Layer 2 access port). | switchport mode access |
| 08 | Define the VLAN membership mode for the port (Layer 2 trunk port). | switchport mode trunk |
| 09 | IP address for the interface | Ip Address x.x.x.x y.y.y.y |