National School of Business Management

LAB Sheet 01

Lab 01 create VLAN and assign two ports to the Vlan

Step1. Create VLAN

Create a VLAN and give an appropriate name to the VLAN

The name must be in the given format

VL01XX (XX is the Group number in two digits eg group 2 will implement VL0102

Step2 Assign IP

- Assign an Interface IP to the created VLAN

Ip interface must be in the following format on IPv4

10.1.X.254 where the group number in single digit  eg group 3 will configure it as 10.1.3.254

Step3 Testing

- Assign two number of ports for the created VLAN (switch access port)

- Connect two PCs to assigned ports (physically connect through UTP cables)

- Do the IP Configurations on the PCs

- Check the connectivity between PCs (ping command)

You  may use the following commands to carry out the task

| No | disruption | command |
| --- | --- | --- |
| 01 | Switch to configuration mode – configuration terminal | Config t |
| 02 | Switch to enable mode of a switch /router | Enable |
| 03 | To go to VLAN configurations | vlan |
| 04 | Change name Name | name |
| 05 | To Access Interface commands | interface |
| 06 | Assign the port to a VLAN. Valid VLAN IDs are 1 to 4094. | switchport access vlan |
| 07 | Define the VLAN membership mode for the port (Layer 2 access port). | switchport mode access |