National School of Business Management

LAB Sheet 04

Lab 04 Static routing

Set up the following scenario on Packet tracer/physical

Note :- please reset  the routers before you start

Step1. Identify the two different ports in the routers

Step2 Assign IP

- Assign an Interface IP to the relevant ports and Ethernet card of the computers

Ip interface must be in the following format on IPv4

10.1.X.250 / where the group number in single digit eg group 3 will configure it as 10.1.3.250

Step3 Testing basic connectivity

- Use the basic ICMP command

Step 4 Adding static routes

- The routers need to be configured to handle manual statis and must be tested if it,s

You may use the following commands to carry out the task

| No | Disruption | command |
| --- | --- | --- |
| 01 | Switch to configuration mode – configuration terminal | Config t |
| 02 | Switch to enable mode of a switch /router | Enable |
| 03 | To go to interface physical | interface FastEthernet 0/0 |
| 04 | Change name Name | name |
| 05 | Prevent the port switch off | No shoutdown |
| 06 | Adding a static rout | ip route (network ip address) (subnet Address) ( next hop Ip address) |